Conceptual study of a thermal storage module for solar power plants with parabolic trough concentrators

V.A. Salomoni1, C.E. Majorana1, G.M. Giannuzzi2, R. Di Maggio3, F. Girardi3, P. Brunello4
1Department of Civil, Environmental and Architectural Engineering, University of Padua, Italy

E-mail: valentina.salomoni@dicea.unipd.it, carmelo.maiorana@dicea.unipd.it

2 ENEA, Agency for New Technologies, Energy and Environment, Thermodynamic Solar Project, CRE Casaccia, Rome, Italy

E-mail: giannuz@casaccia.enea.it

3 Department of Materials Engineering and Industrial Technologies, University of Trento, Italy

E-mail: Rosa.Dimaggio@ing.unitn.it, fabrizio.girardi@ing.unitn.it

4 Department of Technological Physics, University of Padua, Italy 

E-mail: pierfrancesco.brunello@unipd.it

Keywords:  concentrated solar power, trough concentrators, thermal storage.

The thermal storage technology (TSE) has a relevant strategic importance for the success of solar plants devoted to electric energy and heat production. The major benefits in the use of storage include higher efficiency and reduction in the mean levelled cost of the electric energy unit (LEC). 

Sensible heat storage systems within solid media have been identified, both technically and economically, as a very promising solution. The development of such a storage technology, adopting concrete, could reduce the specific cost to less than 20€ per kWh of thermal capacity; additionally, such a solution is suitable for small-medium size plants with a power ranging from 1 MW to 5 MW, to be easily introduced in the Italian territory and with reduced operational and maintenance needs. In large size CSP systems, as the ARCHIMEDE plant built by ENEL with ENEA technology, a high temperature fluid storage (between 400 and 500°C) is required. Such a temperature seems at present not adequate to allow for adopting concrete, whereas the production of concrete able to sustain 250-300°C appears as a reachable objective.

It is supposed to study a storage system characterised by a parallelepiped structure with appropriate section, selfbearing and supported on its major axis, as well as by a piping system directing the thermovector fluid within the cemented matrix [1-3].

Aknowledgments

The Project is partly funded by the Fondazione Cassa di Risparmio di Trento e Rovereto, Prot. SG 2483/10.

References

[1]
Salomoni, V.A.,  Majorana, C.E., Giannuzzi, G.M., Miliozzi, A., "Thermal-fluid flow within innovative heat storage concrete systems for solar power plants", Int. J. Num. Meth. Heat & Fluid Flow (Special Issue), 18(7/8), 969-999 (2008).

[2]
Di Maggio, R., Ischia, G., Bortolotti, M., Rossi, F., Molinari, A., "The microstructure and mechanical properties of Fe-Cu materials fabricated by pressure-less-shaping of nanocrystalline powders", J. Mat. Sci., 42, 9284-9292 (2007).

[3]
Brunello, P., Schrefler, B., Gawin, D., Majorana, C.E., Pesavento, F., "Concrete at High Temperature with Application to Tunnel Fire", Comp. Mech., 29(1), 43-51 (2002).

PAGE  
1

